Mobile Robotics Lab Manual

Lab Session 4: Introduction to ROS 2 Launch, Rosbag, rqt-plot

## 1. Objective

The objective of this lab was to implement a multi-node ROS 2 system using Python-based launch files. Key goals included:

- Automating the startup of a simulation and keyboard control.

- Programming a "Follow-the-Leader" autonomous behavior.

- Using Rosbag to record and verify trajectory patterns.

- Visualizing velocity control commands using rqt_plot.

## 2. Approach & Implementation

### A. Launch Automation

I modified turtlesim_launch.py to handle three tasks in one command:

- Started the turtlesim_node.

- Started the turtle_teleop_key in a separate window using xterm.

- Automatically spawned a second turtle (turtle2) at coordinates (2.0,2.0) using ExecuteProcess to call the /spawn service.

### B. Follower Node Logic

I developed turtle_follower.py, which implements a Proportional (P) Controller:

- Input: Subscribes to /turtle1/pose (Leader) and /turtle2/pose (Follower).

- Process: Calculates the distance error and the heading error.

- Output: Publishes velocity commands to /turtle2/cmd_vel to maintain a target distance.

## 3. Findings & Observations

- Integration: The launch file correctly synchronized all nodes, eliminating the need for manual service calls.

- Controller Stability: The proportional gains (Kp​) chosen for linear and angular velocity allowed turtle2 to track turtle1 smoothly.

- Data Verification: By playing back the recorded bag file (rosbag2_2026_04_18-09_17_41), I confirmed the turtles followed the exact path recorded during the live session, proving the data was captured accurately.